# ETCH Engineering

# DeskHub 1A

# Electrical System Specification

| Rev | Notes | Date |
| --- | --- | --- |
| 0.1 | Initial Draft | 29/07/2026 |

Create something that expands IO from a PC positioned remotely to a unit that would look appropriate on a desk under a monitor, for an enthusiast. There should be appropriate IO, lighting, power delivery and a fan for the user.

Key goals

- USB-C and USB A mixture, primarily 3.2 Gen1.

- Minimum 2 front C ports, 2 rear C ports and 2 rear A ports. 1 upstream C port and barrel jack on the rear.

- All front ports and at least 1 rear port capable of 5V/3A delivery.

- External barrel jack power.

- SD and MicroSD card ports.

- Internal fan to blow air at the user, controlled by a front mounted element.

- Ambient lighting, and status lighting.

Contents

Contents	2

1. Top Level	3

2. USB Architecture	4

1. Top Level

A self-powered desktop USB hub with integrated fan support.

Architecture

Rear

- 1x USB-C Upstream

- 1x DC Power (W TBD)

- 1x USB-C Downstream (3.2G1, 15W)

- 1x USB-C Downstream (3.2G1, 4.5W)

- 2x USB-A Downstream (Min 2.0, 4.5W)

- Power LED

Front

- 2 × USB-C Downstream (3.2G1, 15W)

- Full-size SD

- microSD

- Rotary Encoder with Press (Fan+Light)

Internal

- USB7206

- ESD Protection on all IO, Reverse Polarity Protection on DC Power

- Fused 12V rail for fan and buck supply.

- 5V Buck for USB Supplies

- CC Controller and MUX on all downstream ports (as required)

- USB Power switch on all downstream ports and ancillary electronics.

- 3v3 Reg for USB7206

- 1v15 VCore Reg for USB7206

- Fan for user

- Coordinator MCU (TBD, STM32 Likely) for Fans, LEDs and future expansion.

# 2. Key Components

| Function | Selected Component | Notes |
| --- | --- | --- |
| USB Hub | Microchip USB7206 | 6-port USB 3.2 Gen 1 hub. Chosen for internal expansion. |
| USB SS Mux | TI HD3SS3212 | One per USB-C port. Required for connector orientation. |
| USB Load Switch | TI TPS2553 | One per downstream port. Current limiting and fault reporting. |
| USB ESD | TI TPD4EUSB30DQAR | SuperSpeed-capable, ultra-low capacitance. |
| USB Common Mode Choke | Murata DLW21SN900SQ2L | One per SuperSpeed port. |
| SD Controller | Reserved | USB 3.0 SD/microSD controller. |
| 5 V Regulator |  | Size for 5V ~<15A |
| 3.3 V Regulator | TI TLV76733 | Low-noise LDO for logic rails. |
| Current Monitor (12 V) | TI INA219AIDCNR | Connected to future MCU. |
| Current Monitor (5 V) | TI INA219AIDCNR | Connected to future MCU. |
| Temperature Sensor | TMP102AIDRLR | Reserve footprint if MCU not populated. |
| Crystal | 25 MHz ±20 ppm | Per USB7206 datasheet. |
| Microcontroller | Reserved | Footprint and support circuitry only in Rev A. |

# 3. USB Architecture

USB Hub IC for all 3.2 Gen 2 ports is USB7206.

Every USB-C port shall include:

- USB-C receptacle

- SuperSpeed orientation switch (MUX)

- USB ESD protection

- USB3 common-mode choke

- Per-port power switch

- Power advertisement over CC resistor network.

# 4.Power Architecture

Internal 12V rail is generated downstream of input protection. Lower rails are derived directly from 12V rail.

12V Input

|

Protection

|

12V

|-------- Fan

|-------- 5V USB buck

|-------- 3V3 IC Regulator

|-------- Accessories

Voltage/Current sense on 12V and 5V rails

Input

- 12 V DC Barrel Jack

- Fuse

- TVS

- Reverse polarity protection

- The system may not power on without 12V supplied

5V Rail

- Single synchronous buck

- Current req tbd

5. Hub Controller

Preferred

Microchip USB7206

Configuration

Pin strapping only.

No EEPROM.

Port Allocation

| Hub Port | Function |
| --- | --- |
| 1 | Front USB-C |
| 2 | Front USB-C |
| 3 | Rear USB-C |
| 4 | SD Controller |
| 5 | Internal USB Header |
| 6 | Reserved |

6. SD Card

- Full-size SD

- microSD

Shared controller.

USB connected.

7. Fan

Standard

4-pin PWM PC fan

Signals

- PWM

- Tachometer

Future MCU controls speed.

8. Future MCU

No MCU selected.

Provide:

- Footprint area

- SWD

- UART

- GPIO

- I²C

- ADC inputs

Connections

- Fan PWM

- Fan Tach

- Temperature

- Current sensors

- LEDs

- Hub reset

9. LEDs

Populate footprints for

Mandatory

- Power

- Hub Active

Optional

- Front Port Activity ×2

- Rear Port Activity

- SD Activity

- Overcurrent indicators

- MCU status

10. Temperature Monitoring

Provide footprint for

Digital temperature sensor

or

NTC thermistor

Located near

- Buck regulator

- Hub controller

11. Protection

Every external connector receives ESD protection.

Protected interfaces

- USB-C ×4

- SD

- microSD

- Barrel Jack

12. Internal Expansion

Provide

- Internal USB Header

- 12 V Header

- I²C Header

- UART Header

- GPIO Header

13. Debug

Provide

- Hub Reset

- SWD

- UART

- Test points

Test Points

- VIN

- 12 V

- 5 V

- Hub Reset

- Fan PWM

- Fan Tach

- CC pins (at least one port)

- USB enable lines

- Current monitor outputs

14. PCB

6 Layer

Recommended stack

Top

Ground

Signals

Power

Ground

Bottom

Requirements

- Controlled impedance

- Ground stitching

- Continuous reference planes

- Decoupling adjacent to every IC

15. Hierarchical KiCad Structure

USB_Fan_Hub.kicad_sch

├── Power_Input

├── CoreRegs

├── USB_Hub_Core

├── USB_Upstream

├── USB_Downstream (Front Left)

├── USB_Downstream (Front Right)

├── USB_Downstream (Rear)

├── SD_Reader

├── Fan

├── MCU

└── Debug

17. High-Level System Architecture

12V DC Input

│

Fuse • TVS • Reverse Polarity

│

12V SYSTEM BUS

┌──────────────────────────┼──────────────────────────┐

│                          │                          │

Current Monitor            PWM Fan Header          Future 12V Header

│

└──────────────────────────┬──────────────────────────┐

│                          │

5V Buck Converter             CoreRegs

(≥10 A)                 (3.3 V, etc.)

│

5V Current Monitor

│

5V Distribution Bus

┌───────────────────────┬───────────────┬───────────────────────┐

│                       │               │                       │

USB Switch             USB Switch      USB Switch              USB Hub Power

│                       │               │                       │

Front USB-C #1         Front USB-C #2   Rear USB-C            Microchip USB7206

│                       │               │                       │

MUX + CC + ESD + CMC   MUX + CC + ESD + CMC   MUX + CC + ESD + CMC   │

└───────────────────────┴───────────────┴───────────────────────┤

│

┌───────────────────────────┼─────────────────────────┐

│                           │                         │

USB-C Upstream             SD/microSD Reader      Internal USB Header

│

Reserved Hub Port

Future MCU (reserved footprint)

┌────────────┼─────────────┐

│            │             │

Fan PWM/Tach   LEDs & Status   Current/Temp Sensors

Design Notes

- The design intentionally avoids USB Power Delivery to minimise complexity while remaining USB-C compliant for 5 V/3 A operation.

- All external interfaces are protected with dedicated ESD devices, and all SuperSpeed ports include common-mode chokes to improve EMC performance.

- The USB7206's spare downstream capacity is reserved for internal expansion, allowing future revisions to add features without replacing the hub IC.

- The CoreRegs sheet should provide any auxiliary rails required by the hub, SD controller, and future MCU (for example, 3.3 V or lower-voltage rails if required by the chosen devices). This keeps the power architecture modular and adaptable as component selections are finalised.